DO100 Benchtop DO Meter

#### Features:

- High-resolution LCD display screen.

- With built-in microprocessor chip, automatic calibration, automatic temperature compensation, data storage, parameter setup, etc.

- Automatic calibration, self-diagnosis.

- Fast response, high accuracy, stable reading display.

- Calibration points: DO: 100% Oxygen or Zero oxygen calibration.

- Each kit comes with a polarographic type DO probe and 3pcs replaceable sensor caps.

- IP54 dustproof and spillproof.

#### Specifications:

| DO | Range | 0-20.00 mg/L（ppm）, 0-200.0 % |
| --- | --- | --- |
|  | Accuracy/Resolution | ± 0.30 mg/L； 0.01 or 0.1 mg/L |
|  | ATC range | （0 - 50）℃ automatically |
|  | Salinity compensation | 0~45 ppt manual |
|  | Barometric compensation | 0~200 kPa automatically |
| Temperature | Range | 0-100°C |
|  | Resolution/Accuracy | 0.1℃; ±0.2℃ |
| Data storage |  | 450groups |
| Data Output |  | - USB to Flash Disk - Wireless connection to printer (the printer is optional) |
| Power supply |  | DC12V |
| Packing including |  | - DO100 Meter - DO01-2M Polarographic DO probe, cable length: 2m - DOS-01 Membrane refill solution: 30mL,1pc - DOSC Sensor caps: 3pcs/group, 1pkg - F01 Flexible probe Holder: 1pc - A01 Power adaptor: 1pc |